# MPGuino fuel gauge and trip computer

"MPGuino v0.97u" (MPG version) or "MPGuino v0.97m" (metric version)

Meelis Pärjasaar (meelis @ wiseman.ee), 2012-2014

## Buttons

Buttons from left to right (from top to bottom):

- Previous screen

- Backlight brightness

- Next screen

buttons 1 + 3: Setup screen
buttons 1 + 2: Tank trip reset
buttons 2 + 3: Current trip reset

## Calibration

You should do some calibration to ensure the readings will be accurate.

Calibration instructions are here: http://ecomodder.com/wiki/index.php/Mpguino_calibration

## Settings menu

When you push the buttons 1+3 simultaneously, you can access the Setup menu

- Contrast – LCD Display contrast (default 55, range is 0-255)

- VSS pulses/km or VSS pulses/km - speedometer pulse count when car travels one kilometer. This value is used to calibrate distance and speed.

- MicroSec/Gallon or MicroSec/Litre – micro-seconds how long injector must be open to let through 1 liter of fuel. This value is used to calibrate fuel usage

- Pulses/2 revs - when the mpguino shows wrong RPMs, then here you can change the multiplier (This setting is only displayed for RPM)

- Timout(microSec) – timeout after which the computer stores the trip data (tank trip), and turn itself into sleep mode. (Default = 2 minutes, 120sek = 120 000 000)

- Injector DelayuS - injector nozzle opening delay (default is 500 microseconds)

- Weight (lbs) or Weight (kg) – the car weight in lbs or kilograms (used to calculate the power in DragRace screen)

- Fuel cost *1000 or Fuel EUR/L *1000 - The fuel cost per gallon or liter (used to calculate the cost of current trip)

- VSS Delay ms - speedometer pulse counting delay (default 2 microseconds, reasonable range 0-5)

- InjType 0,1,2 – Injector type. If you press accellerator and in „Car sensors“ scree „Lh“ value is geting smaller, then change this value from 0 to 1 or from 1 to 0. If you press accellerator and this value is not changing almost at all, then maybe your car have „Peak and hold“ type injectors and you could try value 2. 
NB! After changing this setting and leaving settings menu, you must reset mpguino!

- VoltageOffset mV – Using this value you can calibrate voltage gauge. (default 1000 = 1.0V). If real measured value (using voltmeter) is 0.26V smaller, new calibration value is 1000-260=740

## Inputs

Input has 4 wires (counting from top to bottom)

- VSS - speedometer / speed sensor signal

- Inj - one injector signal. Used to measure injector opening time, and can measure the amount of spent fuel

- GND - Ground

- 12V - power input 12V (must not turn off when engine off and key removed)

## Screens

Buttons can switch screens. Different information is displayed in different screens.

### Custom

- Im - Instant Mileage (instant fuel consumption indicator MPG or L/100km). 
When car is not moving then fuel consumption per hour is shown (Gh or Lh).

- T - Current time (current trip time in hours and minutes)

- Cm - Current trip mileage (MPG or L/100km)

- $ - Current Trip cost (unit price can be calibrated in settings)

### Instant / Current

- Im - Instant Mileage (instant fuel consumption indicator MPG or L/100km). 
When car is not moving then fuel consumption per hour is shown (Gh or Lh).

- Spd – Instant speed (mph or km/h)

- Cm - Current trip mileage (MPG or L/100km)

- D - Current Trip Distance

### Current

Current trip screen (must reset manually)

- Sp - average trip speed in mph or km/h

- MG or LK – trip Average Fuel Economy L/100km or MPG

- Mi or km – trip length in miles or kilometers

- Ga or Li – trip fuel spent in gallons or liters

### Tank

Tank trip screen (must reset manually)

- Sp - average trip speed in mph or km/h

- MG or LK – trip Average Fuel Economy L/100km or MPG

- Mi or km – trip length in miles or kilometers

- Ga or Li – trip fuel spent in gallons or liters

### EOC mil/Idle Gal or EOC km/Idle Litr

The EOC is "Engine off coasting" - movement of the car when the engine is turned off or slowing down in-gear when the injectors are closed. These are so-called "free kilometers"

- Ce - Current Trip EOC in miles or kilometers

- G or L - Current Trip idle fuel in gallons or liters (fuel wasted while standing still and engine idling)

- Te - Tank Trip EOC in miles or kilometers

- G or L - Tank Trip idle fuel in gallons or liters (fuel wasted while standing still and engine idling)

### 400m DragRace

Middle button (button 2) resets the previous measuring results. If the car starts moving, time counting starts until 400m has elapsed.

- ET – Elapsed time (400m passing time)

- S – Trap speed (final speed 400m)

- HT – acceleration time 0-100km/h

- HP – car horsepower (calculated from ET and car weight)

### CPU Monitor

MPGuino system information and travel all the time

- CPU% - MPGuino processor load in %

- T - Tank Time (Travel time from the last Tank trip reset. Format is mmm:ss)

- Free MEM - free memory in bytes

### Car Sensors

- V - Battery voltage (battery voltage, charging voltage when engine on)

- DTE - Distance to Empty (Estimated distance you can drive with remaining fuel you have in fuel tank)

- Gh or Lh – fuel consumption (Gallons/hour or Liters/hour)

- RPM - the engine revs

### Graph 18-58 MPG or Graph 3-11L/100k

Here, the display shows eight bars, each one showing 2 minute average fuel consumption. The shorter the column, the lower the average fuel consumption.
In right you can see instant fuel consumption and 2 minute average fuel consumption in numbers (MPG or L/100km).

The graph shows the fuel consumption range:

mpg version: 18-58MPG (40MPG scale, 2.5MPG resolution)

metric version: 3-11L/100km (8L scale, 0.5L resolution)